C181 User Manual

- Product features

- The unique metal appearance design , simple operation

- 800W HD Lens

- Small shape design, portabel hand -held DV

- High definition video recording can be performed under low illumination

- Video resolution:1080P（30fps)  720(30fps)

- Separate recording function

- High speed transmission USB 2.0 port

- Up to 64G TF cards can be supported

- Built -in battery can keep recording for 180-240 minutes

- Structure description

Attention:

- Green light flash prompts low power and automatically stops video recording out of power.

- If there is no TF card in the camera when starting ,blue light will always flash .

- When the TF card is full ,the blue and green lights flash at the same time.

- When there is no operation ,remember to toggle the power switch to shut down it.

Resolution / Loop Video Switching

The product supports 720P (default) and 1080P video recording and loop video recording ( default on ) switch customization,Connect the device to PC with USB or using card reader, create a new document named settings.txt in the root directory of the TF card, and then enter the ( X,X ),X is 0 or 1

The first “ X “ means modifying video resolution:

“0” means 1280*720P  “1” means 1920-1080P

The second “X” means On/Off loop video:

“0” means turning off loop video

“1” means turning on loop video

After the setup is completed ,save the document , start the equipment when the TF card is inserted into the card slot, then can set the video resolution , turn on / off the loop video recording ,and the new TXT document will disappear after the success.

Video:

The mode is switched to the video mode and the blue lightis on, Press the “ Confirm” key once for a short time ,and the bluelight will flash off a few times slowly ,recording video and audio; Press the “Confirm” key again to stop the video recording ,Audio files are automatically saved in segments ,one segment for 5 minute then continuously recorded.

Audio:

The mode is switched to the audio mode and the green light is on . Press the “ Confirm” key once for a short time , and the green light will flash off a few times slowly , recording audio ; Press the “ Confirm” key again to stop the audio recording.

- Operation guide:

Charging:

Built-in rechargeable lithium battery ,please charge first when using it for the first time as follow:

- Use the USB cable to connect PC for charging.

- Video can be recorded while charging( connect USB port 5V charger or mobile power supply)

- The red light is always on when charging ,and goes out when it is full (After the red light goes out , please charge for about 30 minutes to ensure that battery is 100% full)

Connect PC:

Connect to your PC using USB cable ,green light will flash slowly and can read and write TF card , Also it can be used as computer cameras

Time setting:

It can be set by modifying the time in the txt text named “ settime” under the root directory of the TF card .After modifying and saving , disconnect the computer and restart it.

- Product parameter

| Item | Parameter |
| --- | --- |
| Video Format | AVI |
| Video Encoding | Motion JPEG |
| Video resolution | 1980*1080P |
| Video frame rate | 30FPS |
| Media Player | Operating system’s own or mainstream video players |
| Picture format | JPG |
| Image scale | 16:9 |
| System Supported | Windows ME/2000/XP/2003/Vista Mac os Linux |
| Battery capacity | 560mAh |
| Working time | About 180-240 minutes |
| Charging voltage | DC-5V |
| Interface Type | Micro 5 pin sub |
| Storage Support | Up to 64G TF cards can be supported |
| Battery Type | High capacity Polymer lithium battery |